Tomasz Leszczyński

Preliminary parachute design calculations and proposed solution

Required terminal velocity (vt): 5-12 meters per second
Sea level air density (ρ) - 1,225 kg/m3
Estimated mass (m): ~323 grams
Gravitational Constant (g): ~ 9,81 N/kg
Gravitational Force (Fg): 0,31 x 9,81 ~ 3,17 N
Force of gravity must be equal to the force of drag: Fg = FD

Formula used to calculate required surface area: 𝐴 = 2𝐹𝑔𝐶𝐷𝜌𝑣2
Where:
CD – Drag coefficient
v – Velocity

During the calculations we have to take into account the difference in air density on different heights, thus we’ll use a height of 2km to calculate our other limits.

The calculations here do not represent reality perfectly because of how drag coefficient would require prior testing to establish the most adequate value. Used drag coefficient had been sourced from outside materials, for example the one used in calculating the surface area of the chute had been taken as an average of the value given by the CanSat organisers and a document regarding Crossform parachute designs.

Cross parachute:

Much simpler design than a normal semi-spherical one, however it would require a larger surface area due to smaller drag coefficient. The design is also much more stable than a semi-spherical parachute. The parachute would use 4 attachment points on the CanSat due to its shape and also the added benefit of strength.

Drag coefficient ~0.75

At sea level:

For vt = 5 m/s:

𝐴 =2 ⋅ 3,170,75 ⋅ 1,225 ⋅ 25

A = 0,276 m2

For vt = 12 m/s:

𝐴 =2 ⋅ 3,170,75 ⋅ 1,225 ⋅ 144

A = 0,048 m2

At a height of 2000m:

Air density at 2000m – 1,007 kg/m3

For vt = 5 m/s:

𝐴 =2 ⋅ 3,170,75 ⋅ 1,007 ⋅ 25

A = 0,335 m2

For vt = 12 m/s:

𝐴 =2 ⋅ 3,170,75 ⋅ 1,007 ⋅ 144

A = 0,058 m2

Going from these calculations I propose the surface area of A = 0,125 m2 
which would result at a sea level in a terminal velocity of:

𝑣𝑡 = 2𝐹𝑔𝜌𝐴𝐶𝐷 = 2 ⋅ 3,171,225 ⋅0,125 ⋅ 0,75

vt = 7,43 m/s

And at a height of 2000m:

𝑣𝑡 = 2𝐹𝑔𝜌𝐴𝐶𝐷 = 2 ⋅ 3,171,007 ⋅0,125 ⋅ 0,75

vt =  8,19 m/s

This velocity would give us some breathing room whilst still being able to conduct the experiment for a much more prolonged period of time. The number being divisible by 5 also makes the task of sewing the parachute much easier.

The parachute could be made accordingly to these instructions:
http://www.nakka-rocketry.net/xchute1.html
https://www.nakka-rocketry.net/articles/1mxchute.pdf

Accordingly to this and other sources Nylon material should be more than enough for an object of this mass and size dropped from not that high in the atmosphere.